TROUBLESHOOTING NETWORK SERVICES

Q1. Simulate a DNS resolution failure on your local machine by editing the hosts file to point www.spotify.com to a wrong IP address, then try to ping the domain and document the error message you receive.

Ans. Simulate DNS Resolution Failure on Windows

Step 1: Open the hosts file

Open Notepad as Administrator, then open:

C:\Windows\System32\drivers\etc\hosts

Step 2: Add a wrong IP address

At the bottom of the file, add:

192.0.2.1    www.spotify.com

192.0.2.1 is a reserved documentation/test address, so it is suitable for this simulation.

Save the file.

Step 3: Clear the DNS cache

Open Command Prompt as Administrator: ipconfig /flushdns

Expected: Successfully flushed the DNS Resolver Cache.

Step 4: Ping Spotify

Run: ping www.spotify.com

The computer should try to ping 192.0.2.1 instead of Spotify's real server. A typical result may be: Pinging www.spotify.com [192.0.2.1] with 32 bytes of data:

Request timed out.

Ping statistics for 192.0.2.1:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Step 5: Restore the hosts file

Remove the line:

192.0.2.1    www.spotify.com

Then run: ipconfig /flushdns

Result: The hosts-file entry overrides normal DNS resolution and causes www.spotify.com to resolve to the incorrect test IP, resulting in failed ping requests.

Q2. Configure a DHCP server in a network simulator (such as Cisco Packet Tracer or GNS3) and intentionally set an incorrect default gateway; connect a client device and verify the assigned gateway, then describe how you would identify and fix this issue.

Ans. DHCP Server with Incorrect Default Gateway

Step 1: Create the Network
In Cisco Packet Tracer, connect 1 Router, 1 Switch, and 1 PC.

Step 2: Configure DHCP

enable

configure terminal

ip dhcp pool LAN

network 192.168.10.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

exit

- Here, 192.168.20.1 is intentionally set as the wrong default gateway. The router's actual gateway is 192.168.10.1.

Step 3: Get an IP Address
Set the PC to DHCP. It receives:

IP Address:       192.168.10.2

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.20.1

Step 4: Verify the Gateway

On the PC, use: ipconfig

This shows that the PC received the incorrect gateway 192.168.20.1.

Step 5: Identify and Fix the Problem

Check the router configuration:

show running-config

Find: default-router 192.168.20.1

Change it to: default-router 192.168.10.1

Then renew the PC's DHCP address: ipconfig /release, ipconfig /renew

Check again using: ipconfig

The correct gateway should now be: Default Gateway: 192.168.10.1

Conclusion: The incorrect gateway was identified using ipconfig, corrected in the DHCP configuration, and verified after renewing the client's IP address.

Q3. Given a scenario where devices on your simulated network are unable to access websites by name but can by IP address, use the nslookup or dig tool to diagnose the DNS problem and write down the troubleshooting steps and solution.

Ans. DNS Troubleshooting in Simulated Network

Scenario: Devices can access a website using its IP address, but cannot access it using the domain name. This indicates a DNS resolution problem.

Step 1: Test the website by IP address

ping 142.250.183.14

If it replies, network connectivity is working.

Step 2: Test DNS using nslookup

On the PC, run: nslookup www.google.com

If DNS is not working, an error such as “DNS request timed out” or “Server failed” may appear.

Step 3: Check the DNS server configuration

Use: ipconfig /all

Check whether the DNS Server address is correct.

Step 4: Test the DNS server

nslookup www.google.com 8.8.8.8

If this works, the problem is likely with the configured local DNS server.

Step 5: Fix the DNS configuration

Configure the correct DNS server address in the PC/network settings. For example:

Preferred DNS Server: 8.8.8.8

In a Packet Tracer DHCP configuration, use: dns-server 8.8.8.8

Then renew the client's DHCP configuration.

Step 6: Verify the solution

Run: nslookup www.google.com

ping www.google.com

If the domain resolves to an IP address and the ping succeeds, DNS is working correctly.

Conclusion: The problem was caused by an incorrect or unavailable DNS server. Correcting the DNS server address and renewing the client configuration resolved the issue.

Q4. A device in your simulated network is not receiving an IP address from the DHCP server and shows an APIPA address (169.x.x.x). List the possible causes for this and document the step-by-step process you would use to resolve it. Hint: Check for network connectivity, DHCP scope exhaustion, and server availability.

Ans. DHCP Troubleshooting – APIPA Address

Problem: The device is showing an APIPA address such as 169.254.x.x, which means it could not obtain an IP address from the DHCP server.

Possible Causes

- Network cable or connection problem

- DHCP server is down or unavailable

- DHCP scope is exhausted

- Incorrect DHCP configuration

- DHCP request is blocked by a router or firewall

- Device is not configured for DHCP

Troubleshooting Steps

Step 1: Check network connectivity

Check the cable/link between the PC and switch. In Packet Tracer, make sure the link indicators are green.

Step 2: Check the PC configuration

Open: PC → Desktop → IP Configuration

Select DHCP and check whether the PC receives an IP address.

Step 3: Check the DHCP server

Verify that the DHCP server is powered on and the DHCP service is enabled.

Step 4: Check the DHCP scope

Verify that available addresses remain in the DHCP pool. For example:

Network:       192.168.10.0/24

Start IP:      192.168.10.10

End IP:        192.168.10.100

If all addresses are already assigned, expand the scope or release unused leases.

Step 5: Check DHCP configuration

On a Cisco router, use:

show ip dhcp pool

show ip dhcp binding

These commands show the DHCP pool and currently assigned addresses.

Step 6: Renew the IP address

On the PC:

ipconfig /release

ipconfig /renew

Then check: ipconfig

The device should receive an address such as:

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

Conclusion: The APIPA address was caused by the device failing to communicate with the DHCP server. After checking connectivity, DHCP availability, and scope capacity, renewing the DHCP lease should provide a valid IP address.

Q5. Create a table documenting at least three common DNS issues and three common DHCP issues you encountered or researched, including symptoms, possible causes, and recommended solutions for each.

Ans. Common DNS and DHCP Issues

| Type | Issue | Symptoms | Possible Cause | Recommended Solution |
| --- | --- | --- | --- | --- |
| DNS | DNS server unavailable | Websites do not open by name, but work by IP | DNS server is down or unreachable | Check DNS server and network connection |
| DNS | Wrong DNS record | Domain resolves to the wrong IP | Incorrect A/CNAME record | Check and correct the DNS record |
| DNS | DNS cache problem | Old or incorrect IP address is returned | Outdated DNS cache | Run ipconfig /flushdns and test with nslookup |
| DHCP | DHCP server unavailable | Device gets 169.254.x.x APIPA address | DHCP server is down or unreachable | Check server availability and network connection |
| DHCP | DHCP scope exhausted | Device cannot obtain an IP address | All available IP addresses are used | Expand the DHCP scope or release unused leases |
| DHCP | Incorrect gateway | Local devices may work, but other networks cannot be reached | Wrong default-router in DHCP configuration | Correct the gateway and renew the DHCP lease |