Introduction to ip addressing

Q1. List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each.

Ans.

| IPv4 | IPv6 |
| --- | --- |
| Uses 32-bit addresses | Uses 128-bit addresses |
| Written in decimal, e.g., 192.168.1.10 | Written in hexadecimal, e.g., 2001:db8::1 |
| Provides about 4.3 billion addresses | Provides a vastly larger number of addresses |
| Uses ARP to find MAC addresses | Uses Neighbour Discovery Protocol (NDP) |
| Supports broadcast communication | Has no broadcast; uses multicast instead |
| Usually requires NAT due to address shortage | Generally does not require NAT because of abundant addresses |

Q2. Given the following scenario: You have 4 devices (laptop, phone, smart TV, and printer) connected to your home Wi-Fi. Assign a unique static IPv4 address to each device using the 192.168.10.0/24 network, and write down the IP, subnet mask, and default gateway for each device.

Ans. Static IPv4 Address Assignment

Network: 192.168.10.0/24
Subnet Mask: 255.255.255.0
Default Gateway: 192.168.10.1

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Phone | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |

Note: 192.168.10.0 is the network address and 192.168.10.255 is the broadcast address, so they should not be assigned to devices.

Q3. Calculate the number of valid subnets and hosts per subnet if you use a /27 CIDR subnet mask on a 192.168.1.0 network. List the first three valid subnet address ranges (network address to broadcast address) and the usable host IPs in each.

Ans. For 192.168.1.0/27:

- Subnet mask: 255.255.255.224

- Number of subnets: 8

- Usable hosts per subnet: 30

- Total addresses per subnet: 32

| Subnet | Network Address | Usable Host IPs | Broadcast |
| --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 – 192.168.1.30 | 192.168.1.31 |
| 2 | 192.168.1.32 | 192.168.1.33 – 192.168.1.62 | 192.168.1.63 |
| 3 | 192.168.1.64 | 192.168.1.65 – 192.168.1.94 | 192.168.1.95 |

Calculation: /27 leaves 5 host bits → 2⁵ = 32 addresses per subnet → 32 − 2 = 30 usable hosts.

Q4. You are given the IP address 10.0.5.17 with subnet mask 255.255.255.0. Determine the network address, broadcast address, and the range of valid host addresses for this subnet. Hint: Convert the subnet mask to CIDR notation and use bitwise AND for network address calculation.

Ans. Given:

- IP Address: 10.0.5.17

- Subnet Mask: 255.255.255.0 = /24

| Item | Answer |
| --- | --- |
| Network Address | 10.0.5.0 |
| Broadcast Address | 10.0.5.255 |
| Valid Host Range | 10.0.5.0 – 10.0.5.254 |
| Usable Host | 254 |

Bitwise AND: 10.0.5.17 AND 255.255.255.0 = 10.0.5.0

So, the subnet is 10.0.5.0/24, with valid host addresses from 10.0.5.1 to 10.0.5.254.

Q5. Use an online subnet calculator tool (such as https://www.subnet-calculator.com/) to verify your answer for the previous task. Take a screenshot of your calculation and write one line explaining if your manual answer matched the tool's output.

Ans. My manual answer matched the subnet calculator's output exactly.