SUBNETTING PRACTICAL

Q1. Given a network address of 192.168.10.0/24, divide it into 4 equal subnets and write down the subnet mask, network address, and broadcast address for each subnet.

Ans. To divide 192.168.10.0/24 into 4 equal subnets, borrow 2 host bits:

New subnet mask: /26 = 255.255.255.192

| Subnet | Network Address | Broadcast Address | Usable Host Range |
| --- | --- | --- | --- |
| 1 | 192.168.10.0/26 | 192.168.10.63 | 192.168.10.1-192.168.10.62 |
| 2 | 192.168.10.64/26 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |
| 3 | 192.168.10.128/26 | 192.168.10.191 | 192.168.10.129 – 192.168.10.190 |
| 4 | 192.168.10.192/26 | 192.168.10.255 | 192.168.10.193 – 192.168.10.254 |

Q2. For one of the subnets you created, assign valid IP addresses to 5 devices (such as a router, two laptops, a printer, and a mobile) and list which IP goes to which device. Hint: Make sure not to use the network or broadcast address for any device.

Ans. Let's use Subnet 1: 192.168.10.0/26.

- Network address: 192.168.10.0

- Broadcast address: 192.168.10.63

- Valid host range: 192.168.10.1 – 192.168.10.62

| Device | Assigned IP Address |
| --- | --- |
| Router | 192.168.10.1 |
| Laptop 1 | 192.168.10.2 |
| Laptop 2 | 192.168.10.3 |
| Printer | 192.168.10.4 |
| Mobile | 192.168.10.5 |

Subnet Mask: 255.255.255.192

All five addresses are valid because they are between 192.168.10.1 and 192.168.10.62 and do not use the network or broadcast address.

Q3. Draw a simple diagram (hand-drawn or digital) showing your subnet plan: label each subnet, the range of IPs, and where each device is connected.

Ans.

Q4. Document step-by-step how you calculated the subnet mask, number of hosts per subnet, and assigned IP addresses for your plan, as if you are explaining it to a friend who is new to subnetting.

Ans. Step 1: Calculate the Subnet Mask

- Network: 192.168.1.0/24

- Subnet mask: 255.255.255.0

- We need 4 subnets, so we borrow 2 bits.

- New subnet mask: /26 = 255.255.255.192

Step 2: Calculate Hosts per Subnet

- Remaining host bits: 6

- Total addresses: 2⁶ = 64

- Usable hosts: 64 − 2 = 62

- So, each subnet supports 62 usable devices.

Step 3: Assign IP Addresses

For the first subnet:

- Network address: 192.168.1.0

- Router: 192.168.1.1

- Laptop 1: 192.168.1.2

- Laptop 2: 192.168.1.3

- Printer: 192.168.1.4

- Mobile: 192.168.1.5

- Broadcast address: 192.168.1.63

Subnet mask for all devices: 255.255.255.192