IP ADDERRSING AND SUBNETTING

Q1. List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

Ans.

| IP Class | Address Range | Purpose | Real-World Examples |
| --- | --- | --- | --- |
| Class A | 1– 126 | Large networks | A large organization or ISP using a private network such as 10.0.0.0/8 |
| Class B | 128– 191 | Medium-sized networks | A university network using an address such as 172.16.0.0/16 |
| Class C | 192– 223 | Small networks | A home/office LAN using 192.168.1.0/24 |
| Class D | 224– 239 | Multicast communication | Video/audio streaming or multicast services |
| Class E | 240– 255 | Experimental/reserved | Used for research and testing, not normally assigned to everyday devices |

Q2. Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses. Hint: Show your working steps for each calculation.

Ans. Step 1: Calculate the Network Address

Perform an AND operation between the IP address and subnet mask:

IP Address:   192.168.10.45

Subnet Mask:  255.255.255.0

Network:      192.168.10.0

Network Address = 192.168.10.0

Step 2: Calculate the Broadcast Address

For a /24 network, the last 8 bits are available for hosts. Set all host bits to 1:

192.168.10.0 →  192.168.10.255

Broadcast Address = 192.168.10.255

Step 3: Calculate Valid Host Range

The first address is reserved for the network and the last for broadcast:

Network:     192.168.10.0

First Host:  192.168.10.1

Last Host:   192.168.10.254

Broadcast:   192.168.10.255

Valid Host Addresses = 192.168.10.1 – 192.168.10.254

Q3. Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

Ans. Subnetting Plan for a Food Delivery Startup

We have 10.0.0.0/24 and need 4 separate subnets.
To create 4 equal subnets, we borrow 2 host bits:

/24 → /26

- Subnet mask: 255.255.255.192

- Each subnet has 64 addresses

- 62 usable host addresses per subnet

| Group | Network Address | Subnet Mask | Usable Host Range | Broadcast |
| --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.255/26 | 255.255.255.192 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

Q4. You are given the following scenario: A college Wi-Fi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet. Constraint: Show your calculation steps and final answers.

Ans. Step 1: Calculate the number of bits to borrow

We need to find n such that:

2ⁿ ≥ 20

- 2⁴ = 16 (not enough)

- 2⁵ = 32

So, we need to borrow 5 bits from the host portion.

Step 2: Calculate the new subnet mask

Original prefix: /16

Borrowed bits: 5

New prefix: 16 + 5 = /21

The subnet mask for /21 is:

255.255.248.0

Step 3: Calculate hosts per subnet

/21 leaves: 32 − 21 = 11 host bits

Total addresses: 2¹¹ = 2048

Usable host addresses: 2048 − 2 = 2046

(We subtract the network and broadcast addresses.)

Final Answer

| Item | Answer |
| --- | --- |
| Original Network | 172.16.0.0/16 |
| Required Subnets | 20 |
| Bits Borrowed | 5 |
| Numbers of Subnets Created | 32 |
| New Subnet Mask | 255.255.248.0 |
| New Prefix | /21 |
| Total Address/Subnet | 2048 |
| Usable Hosts/Subnet | 2046 |

- Final: Use a /21 subnet mask (255.255.248.0), providing 32 subnets with 2046 usable hosts per subnet.

Q5. A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

Ans. The problem is that 192.168.1.300 is not a valid IPv4 address. Each part of an IPv4 address must be between 0 and 255, but 300 is greater than 255.

If the device is on the 192.168.1.0/24 subnet, a valid host IP could look like 192.168.1.100. The usual valid host range is 192.168.1.1 to 192.168.1.254.