Network Security, Maintenance & Troubleshooting Procedures

SECTION – A

Q1. What factors should be considered when selecting the CPU, RAM, and storage to ensure good performance and hardware compatibility?

Ans. When selecting CPU, RAM, and storage, consider these factors for good performance and compatibility:

| Component | Factors to considered |
| --- | --- |
| CPU | Processor generation, number of cores/threads, clock speed, performance requirements, motherboard socket, and power consumption. |
| RAM | Capacity (8 GB, 16 GB, 32 GB, etc.), RAM type (DDR4/DDR5), speed, motherboard compatibility, and number of available RAM slots. |
| Storage | SSD vs HDD, storage capacity, read/write speed, interface (SATA/NVMe), and motherboard support for the drive. |

Other important factors

- Motherboard compatibility: CPU socket, RAM type, and storage interfaces must be supported by the motherboard.

- Power supply: The PSU should provide enough power for the CPU and other components.

- Cooling: A powerful CPU may require better cooling.

- Usage: Gaming, video editing, programming, or basic office work require different hardware levels.

- Future upgrades: Choose components that leave room for adding more RAM or storage later.

Q2. Explain how the antivirus software affects CPU, RAM, and storage performance, leading to system slowdowns.

Ans. Antivirus software can slow down a computer because it continuously monitors files, programs, and network activity.

- CPU: Antivirus scans files and running programs, using CPU processing power. Full system scans can cause higher CPU usage and make other programs slower.

- RAM: Antivirus programs run background services and keep security databases in memory, using some RAM. If RAM is limited, this can reduce memory available for other applications.

- Storage: Antivirus frequently reads and scans files on the HDD/SSD. During large scans, heavy storage activity can slow down file access and application loading.

- Overall slowdown: When CPU, RAM, and storage are heavily used at the same time, the system may become slower, especially on older computers or systems with limited resources.

Example: If an antivirus performs a full scan while you are gaming, it may use CPU and storage resources, potentially causing longer loading times or reduced game performance.

Q3. Describe the steps you would follow to troubleshoot the problem if it is caused by driver incompatibility or operating system issues.

Ans. Steps to Troubleshoot Driver or Operating System Issues

- Identify the problem: Check which hardware or application is causing the issue and note any error messages.

- Restart the computer: A restart can fix temporary driver or OS problems.

- Check Device Manager: Look for devices with a warning or error symbol.

- Update drivers: Download and install the latest compatible driver from the hardware manufacturer's website.

- Roll back drivers: If the problem started after a driver update, restore the previous driver version.

- Install OS updates: Check Windows Update and install available security and system updates.

- Run system checks: Use tools such as System File Checker (SFC) and DISM to repair corrupted Windows system files.

- Check compatibility: Make sure the driver is designed for the installed Windows version and hardware.

- Use System Restore: If the issue continues, restore Windows to a previous working state.

- Test the system: Restart the computer and check whether the problem has been resolved.

Q4. Explain how a UPS and surge protector help protect computer hardware and prevent data loss during these situations.

Ans. UPS and Surge Protector

- UPS (Uninterruptible Power Supply): A UPS provides temporary battery power when the electricity goes out. This gives you time to save your work and shut down the computer safely, helping prevent data loss and damage caused by sudden power failure. It can also help stabilize voltage.

- Surge Protector: A surge protector protects computer components from sudden voltage spikes or electrical surges. It diverts excess voltage away from the computer, protecting the power supply, motherboard, and other hardware.

Q5. Explain how DNS and DHCP work together in this network, and identify which service is most likely misconfigured based on the symptoms described.

Ans. DNS and DHCP Working Together

- DHCP (Dynamic Host Configuration Protocol) automatically provides devices with network settings such as an IP address, subnet mask, default gateway, and DNS server address.

- DNS (Domain Name System) converts website names such as google.com into their corresponding IP addresses, allowing the device to connect to the website.

- How They Work Together

- A device connects to the network.

- DHCP assigns it an IP address and tells it which DNS server to use.

- When the user enters a website name, the device sends a request to the DNS server.

- DNS finds the website's IP address.

- The device connects to that IP address through the gateway.

- Identifying the Problem

- If the symptoms are that websites can be accessed using their IP addresses but not by their names, the DNS service is most likely misconfigured.

- If devices are not receiving valid IP addresses or network settings, then DHCP is more likely to be misconfigured.

- Example: If 8.8.8.8 works but google.com does not, the problem is most likely DNS.

Q6. Describe the troubleshooting steps you would take to determine whether the firewall rules are blocking legitimate traffic, and how you would resolve the issue.

Ans. Firewall Troubleshooting Steps

- Identify the problem: Check which application, port, or service is unable to communicate.

- Check firewall rules: Open the firewall settings and review inbound and outbound rules for blocked traffic.

- Check firewall logs: Look for blocked connections and note the IP address, port, and application involved.

- Test the connection: Temporarily disable the firewall for a short test to determine whether it is causing the problem. Re-enable it immediately after testing.

- Check the rule: Verify that the blocked port, protocol, or application is actually required for legitimate traffic.

- Modify or create a rule: If the traffic is legitimate, create a specific allow rule for the required application, port, or network rather than allowing all traffic.

- Remove unnecessary rules: Delete or disable conflicting or outdated blocking rules.

- Test again: Restart the application or service and confirm that the connection works.

- Monitor: Check the firewall logs again to ensure legitimate traffic is no longer being blocked.

In short: Identify the blocked traffic, verify it is legitimate, adjust the specific firewall rule, and test the connection again while keeping unnecessary traffic blocked.

SECTION – B

Task 1. Workstation Assembly

Install CPU, RAM, storage, and other core components into the chassis in the correct sequence. Connect all required internal cables (power, data, front panel). Document each installation step in order. System must pass POST (Power-On Self-Test) after assembly.

Ans. PC Assembly: Installation Steps

- Follow these steps in order to assemble the computer and ensure it successfully passes POST (Power-On Self-Test).

- Prepare the workspace

  - Turn off and unplug the power supply.

  - Use an anti-static wrist strap or regularly ground yourself.

  - Place all components and screws nearby.

- Install the CPU

  - Place the motherboard on an anti-static surface.

  - Open the CPU socket.

  - Align the CPU's marker with the socket marker.

  - Carefully place the CPU into the socket and secure the retention mechanism.

- Install CPU cooling

  - Apply thermal paste if the cooler does not already have it.

  - Place the CPU cooler over the CPU.

  - Secure it firmly and connect the cooler's fan cable to the CPU_FAN header.

- Install RAM

  - Open the motherboard's RAM slot clips.

  - Align the notch on the RAM module with the slot.

  - Press down evenly until the clips lock into place.

  - Use the motherboard's recommended slots for multiple RAM modules.

- Install storage

  - For an M.2 SSD, insert it into the appropriate M.2 slot and secure it with the screw.

  - For a SATA SSD/HDD, mount the drive in its bay and connect the SATA data cable.

- Install the motherboard

  - Install the correct I/O shield if required.

  - Place the motherboard onto the chassis standoffs.

  - Secure it using the appropriate screws.

- Install the power supply

  - Mount the PSU in its designated position.

  - Connect the 24-pin ATX motherboard power cable.

  - Connect the 8-pin (or required) CPU power cable.

- Install the graphics card and other expansion cards

  - Insert the graphics card into the appropriate PCIe slot.

  - Secure it to the chassis.

  - Connect its additional PCIe power cable if required.

- Connect storage cables

  - Connect SATA data cables between SATA drives and the motherboard.

  - Connect SATA power cables from the PSU to the drives.

- Connect front-panel cables

- Connect Power SW, Reset SW, Power LED, and HDD LED according to the motherboard manual.

- Connect front USB and front audio cables to their appropriate headers.

- Check all connections

- Ensure RAM, CPU cooler, GPU, and cables are properly seated.

- Make sure no loose screws or cables are interfering with fans.

- Perform the first POST test

- Connect the monitor, keyboard, and power cable.

- Turn on the PSU and press the computer's power button.

- Check for display output and listen for beep codes or check motherboard diagnostic LEDs.

- Verify POST

- Enter the BIOS/UEFI using the indicated key, commonly Delete or F2.

- Confirm that the CPU, RAM, and storage drive are detected.

- If everything is detected without POST errors, the assembly has successfully passed POST.

Final check: The system should power on normally, display the POST/BIOS screen, and correctly recognize the installed CPU, RAM, and storage.

Task 2. RAM Mismatch Troubleshooting

Identify the RAM mismatch error via BIOS/UEFI POST messages. Determine the root cause (mismatched speed, timing, or capacity). Resolve the mismatch by reconfiguring or reseating the RAM correctly. Confirm the system boots normally after the fix.

Ans. Troubleshooting RAM Mismatch Error

- Check the POST message

  - Turn on the computer and observe the BIOS/UEFI screen.

  - Look for messages such as “Memory Error,” “RAM Error,” “Memory Mismatch,” or “Memory Training Failed.”

  - Check the motherboard's diagnostic LEDs or beep codes if available.

- Check RAM in BIOS/UEFI

  - Enter BIOS/UEFI using Delete, F2, or the key specified by the motherboard.

  - Check the detected RAM capacity, frequency/speed, and timings.

- Determine the root cause

  - Speed mismatch: RAM modules have different supported speeds, or the selected XMP/EXPO speed is not stable.

  - Timing mismatch: RAM modules have different latency/timing specifications.

  - Capacity mismatch: Modules have different capacities or are installed in an unsupported configuration.

  - Also check whether the RAM type is supported by the motherboard.

- Power off and reseat the RAM

  - Shut down the computer and disconnect the power.

  - Remove the RAM modules.

  - Check for dust or visible damage.

  - Reinstall the modules firmly until the locking clips click into place.

  - Use the motherboard's recommended slots, usually A2 and B2 for two modules.

- Reconfigure the RAM

  - Enter BIOS/UEFI again.

  - Disable XMP/EXPO temporarily and use the motherboard's default memory settings.

  - If necessary, set a common supported speed and timings for all installed modules.

- Test the modules

  - Boot with one RAM module at a time to identify a potentially faulty or incompatible module.

  - If possible, test the RAM using a memory diagnostic tool.

- Confirm the fix

  - Restart the computer and check that the POST error no longer appears.

  - Enter BIOS/UEFI and verify that the full RAM capacity is detected.

  - Boot into Windows and confirm the system operates normally.

Result: The RAM mismatch is resolved when the modules are correctly seated and configured with compatible capacity, speed, and timings, and the computer completes POST and boots normally.

Task 3. LAN Topology Build

Use Cisco Packet Tracer to build the topology. Include 5 end-systems and 1 network printer. Assign IP addresses from a Class C subnet to all devices. All assigned IPs must be reachable via Ping.

Ans.

- Here I have assigned IP to all devices with the help of server and they all are communicating with each other

Task 4. Clean OS Installation

Perform a clean install of Windows/Linux from ISO. Manually partition the disk (not guided/automatic partitioning). Optimize startup programs/services for faster boot. OS must boot without errors after installation.

Ans. Clean OS Installation and Startup Optimization

1. Prepare the installation media

- Download the required Windows or Linux ISO.

- Create a bootable USB using a tool such as Rufus or a Linux USB creator.

- Back up important files because a clean installation can erase existing data.

- Restart the computer and boot from the USB using the BIOS/UEFI boot menu.

2. Start the clean installation

- Select the language, keyboard, and installation options.

- Choose Custom/Manual installation instead of Upgrade or Automatic/Guided installation.

- Select the target disk and manually create the required partitions.

Example for a UEFI system:

| Partition | Suggested Size | Purpose |
| --- | --- | --- |
| EFI System Partition | 100–500 MB | Boot files |
| System/Root | 50–100+ GB | Operating system |
| Swap | Optional | Linux virtual memory |
| Data/Home | Remaining space | User files |

- Important: Partition layouts differ between Windows and Linux. Do not delete existing partitions unless you have confirmed they are safe to erase.

3. Install the OS

- Select the manually created system partition as the installation destination.

- Continue the installation and allow the computer to restart.

- Remove the USB when the installer finishes or ensure the system boots from the internal drive.

4. Install updates and drivers

- Complete the initial setup.

- Install the latest OS updates.

- Install required hardware drivers such as chipset, graphics, network, and audio drivers.

- Restart the computer and check for errors.

5. Optimize startup

- Open the startup-app management tool:

- Windows: Task Manager → Startup Apps.

- Linux: Use the appropriate desktop's Startup Applications tool or systemd services.

- Disable unnecessary applications that automatically start.

- Avoid disabling essential system services, security software, hardware drivers, or networking services.

- Restart the computer and compare the boot time.

6. Verify the installation

Confirm that:

- The OS boots directly from the internal disk without errors.

- The manually created partitions are detected correctly.

- CPU, RAM, storage, network, audio, and display work properly.

- No important devices have driver errors.

- Unnecessary startup programs have been disabled.

- The system reaches the desktop normally after several restarts.

Final result: The computer should have a clean OS installation with manually configured partitions and a streamlined startup configuration, while maintaining all essential system services.

SECTION – C

Q1. Identify the three hardware components most likely to cause random reboots under load.

Ans. The three hardware components most likely to cause random reboots under heavy load are:

- Power Supply Unit (PSU)

  - An insufficient, faulty, or overheating PSU may fail to provide stable power when the CPU/GPU is under heavy load.

  - This can cause sudden restarts or shutdowns.

- CPU

  - An overheating CPU can trigger thermal protection and automatically restart or shut down the system.

  - Check CPU temperatures and make sure the cooler and thermal paste are installed correctly.

- RAM

  - Faulty, incompatible, or incorrectly configured RAM can cause system instability and unexpected reboots.

  - Run a memory diagnostic test and check that the RAM is properly seated.

Other possible causes: A faulty graphics card, overheating GPU, motherboard problems, or inadequate cooling can also cause random restarts.

Q2. Explain why the systems pass POST in the morning but fail in the afternoon.

Ans. Systems may pass POST in the morning but fail in the afternoon mainly because the temperature and power conditions can change during the day.

- Higher temperature: Afternoon room temperatures are often higher. CPU, RAM, or motherboard components may become hotter, causing instability during POST.

- Overheating: Dusty fans, blocked vents, or poor thermal paste can make cooling less effective as temperatures rise.

- Power fluctuations: Changes in the electrical supply or increased load on the power circuit can cause unstable voltage, affecting the PSU and motherboard.

- Faulty components: A failing PSU, RAM, or motherboard may work when cool but become unstable as it heats up.

- Poor ventilation: A computer in a closed or poorly ventilated area can accumulate heat throughout the day.

Troubleshooting: Check CPU/system temperatures, clean the cooling system, verify PSU voltage stability, reseat the RAM, and test individual components. If the problem occurs mainly when the system is warm, thermal or power-related hardware problems are particularly worth investigating.

Q3. Describe how you would use a stress-test tool to confirm a thermal or power issue.

Ans. Using a Stress Test to Confirm Thermal or Power Issues

- Monitor temperatures first: Use a hardware-monitoring tool such as HWMonitor or HWiNFO to record the CPU and GPU temperatures at idle.

- Run a CPU stress test: Use a tool such as Prime95 or OCCT and run it for around 10–15 minutes while monitoring CPU temperature and system stability.

- Run a GPU stress test: Use a tool such as FurMark or OCCT to place a heavy load on the GPU. Monitor temperature and power usage.

- Test combined load: If the system remains stable during individual tests, run a CPU + GPU load together. This places greater demand on the PSU.

- Observe the symptoms:

  - Sudden shutdown/reboot under heavy load → possible PSU or power-delivery problem.

  - Temperatures rapidly reaching unsafe levels before shutdown → likely cooling/thermal problem.

  - Errors or crashes without excessive temperature → could indicate RAM, CPU, GPU, or motherboard instability.

- Compare results: Repeat the test after cleaning the system or improving cooling. If the problem disappears after temperatures are reduced, that supports a thermal cause.

- Stop if temperatures become unsafe: Do not continue a stress test if temperatures approach the component manufacturer's maximum limits or the system becomes unstable.

Conclusion: A stress test combined with temperature and power monitoring helps determine whether the random reboot is caused primarily by overheating or inadequate/unstable power delivery.

Q4. Propose three specific steps to remediate the frequent system restarts.

Ans. Three Steps to Remediate Frequent System Restarts

- Improve cooling

  - Clean dust from the CPU/GPU heatsinks and fans.

  - Ensure all fans are working and airflow is unobstructed.

  - Replace thermal paste if necessary and verify CPU/GPU temperatures.

- Check or replace the PSU

  - Make sure the PSU has sufficient wattage for the system.

  - Check power cables and connections.

  - If the PSU is old or unstable, replace it with a reliable, appropriately rated unit.

- Test and reseat RAM

  - Power off the PC and reseat the RAM modules.

  - Test each RAM module individually.

  - Run a memory diagnostic test and replace any faulty or incompatible RAM.

- After each step, run the system under normal/heavy load and check whether the restarts continue.

Q5. Provide a brief technical justification for why a PSU upgrade might (or might not) be necessary.

Ans. Technical Justification for a PSU Upgrade

- A PSU upgrade may be necessary if the current power supply cannot provide enough stable power for the CPU and GPU, especially during heavy workloads. An overloaded or failing PSU can cause random restarts, shutdowns, or crashes when power demand suddenly increases.

- A PSU upgrade may not be necessary if the existing PSU has sufficient wattage, good voltage stability, and is in good condition. In that case, frequent restarts may instead be caused by overheating, faulty RAM, GPU problems, or motherboard issues.

Conclusion: Check the system's maximum power requirements, PSU capacity, age, condition, and voltage stability before replacing it. A higher-wattage PSU alone will not fix a problem if the actual cause is another component.

SECTION – D

Step 1. Build (With AI)

The AI-generated plan must specify the subnet, subnet mask, usable host range, and broadcast address. The plan must allocate addresses for all 5 systems and 1 printer named in the scenario. Instruct the AI to also identify which addresses to reserve for future expansion. Document the exact prompt(s) used.

Ans. Exact Prompt Given to AI

Generate an IP addressing and subnetting plan for Nexus Innovations' startup office network. The network must support 5 systems and 1 network printer. Use a private IPv4 Class C network. Specify the subnet address, subnet mask, usable host range, and broadcast address. Assign a unique IP address to each of the 5 systems and the printer. Also identify which IP addresses should be reserved for future expansion. Present the result in a clear table and make sure there is enough host capacity for all current devices and future growth.

2. AI Answer — Original Plan

Subnet: 192.168.10.0/24
Subnet Mask: 255.255.255.0

| Device | IP Address |
| --- | --- |
| System 1 | 192.168.10.1 |
| System 2 | 192.168.10.2 |
| System 3 | 192.168.10.3 |
| System 4 | 192.168.10.4 |
| System 5 | 192.168.10.5 |
| Printer | 192.168.10.6 |

- Network Address: 192.168.10.0

- Usable Host Range: 192.168.10.1 – 192.168.10.254

- Broadcast Address: 192.168.10.255

- Reserved for Future Expansion: 192.168.10.7 – 192.168.10.254

Step 2. Test And Debug (Without AI)

Then, working without AI, verify the plan for errors — overlapping ranges, incorrect network/broadcast addresses, insufficient host capacity, or unreserved growth space — and correct them yourself.

Ans. I manually checked the network address, broadcast address, host capacity, and future growth space.

Subnet: 192.168.10.0/28
Subnet Mask: 255.255.255.240

| Device | IP Address |
| --- | --- |
| System 1 | 192.168.10.2 |
| System 2 | 192.168.10.3 |
| System 3 | 192.168.10.4 |
| System 4 | 192.168.10.5 |
| System 5 | 192.168.10.6 |
| Printer | 192.168.10.7 |

- Network Address: 192.168.10.0

- Usable Host Range: 192.168.10.1 – 192.168.10.14

- Broadcast Address: 192.168.10.15

- Reserved for Future Expansion: 192.168.10.8 – 192.168.10.14

- Manual Verification Note

- I manually checked the AI plan for overlapping addresses, incorrect network/broadcast addresses, and host capacity. The AI plan was technically correct, but a /24 subnet was unnecessarily large for only 6 devices. I changed it to /28, which provides 14 usable addresses and keeps 7 addresses available for future expansion. This makes the network more suitable for a small startup office.

- Final Difference

- AI: /24 → 254 usable addresses

- Manual: /28 → 14 usable addresses, with 7 reserved for future expansion.